# Study Notes: Application Layer (ITN Module 15)

## 1. Application, Presentation, and Session Layers

### Application Layer (Layer 7)

- Provides an interface between applications and the network.

- Uses well-known protocols: HTTP, FTP, TFTP, IMAP, DNS.

### Presentation Layer (Layer 6)

- Functions:

  - Formatting data into a compatible format for the destination device.

  - Compressing data to optimize transmission.

  - Encrypting data for security and decrypting it upon arrival.

### Session Layer (Layer 5)

- Functions:

  - Creates and maintains communication sessions between applications.

  - Manages dialogs between source and destination applications.

  - Resumes communication if interrupted.

2. Peer-to-Peer (P2P) Networking

### Client-Server Model

- Client: Requests services or data.

- Server: Provides requested data/services.

- Application layer protocols define request and response formats.

### P2P Network Characteristics

- Computers act as both clients and servers.

- No central server required; devices share resources directly.

### P2P Applications

- Hybrid P2P: Uses a central index server to locate resources.

- Examples:

  - BitTorrent

  - Direct Connect

  - eDonkey

## 3. Web and Email Protocols

### Hypertext Transfer Protocol (HTTP/HTTPS)

- HTTP (port 80): Standard for web browsing.

- HTTPS (port 443): Secure version of HTTP with encryption.

- Message Types:

  - GET – Requests data from a web server.

  - POST – Sends data (e.g., form submission).

  - PUT – Uploads resources to the web server.

### Email Protocols

- SMTP (Simple Mail Transfer Protocol)

  - Used for sending email (port 25).

  - If the recipient is offline, the email is stored and retried later.

- POP (Post Office Protocol)

  - Used for retrieving email (port 110).

  - Downloads and deletes messages from the mail server.

  - Not ideal for businesses needing centralized storage.

- IMAP (Internet Message Access Protocol)

  - Retrieves emails while keeping copies on the server (port 143).

  - Synchronizes changes (e.g., deleting emails updates the server).

## 4. IP Addressing Services

### Domain Name System (DNS)

- Converts domain names into IP addresses.

- Fully Qualified Domain Names (FQDNs) are used (e.g., www.cisco.com).

- Record Types:

  - A – IPv4 address.

  - AAAA – IPv6 address.

  - MX – Mail exchange record.

  - NS – Authoritative name server.

### DNS Hierarchy

- Uses a distributed database for name resolution.

- Top-Level Domains (TLDs):

  - .com (business)

  - .org (non-profits)

  - .au (Australia)

### nslookup Command

- Used to manually query DNS servers and troubleshoot name resolution.

### Dynamic Host Configuration Protocol (DHCP)

- Automates assignment of IPv4 addresses.

- Assigns subnet masks, gateways, and DNS settings.

- Uses a lease system (temporary IP allocation).

- DHCPv6 supports IPv6 but does not assign default gateways.

### DHCP Process

- DHCPDISCOVER – Client requests an IP.

- DHCPOFFER – Server offers an available IP.

- DHCPREQUEST – Client selects an offer.

- DHCPACK – Server confirms and assigns the IP.

## 5. File Sharing Services

### File Transfer Protocol (FTP)

- Used for transferring files between a client and a server.

- Uses two connections:

  - Control connection (port 21) – Handles commands.

  - Data connection (port 20) – Transfers files.

### Server Message Block (SMB)

- File-sharing protocol allowing access to remote files.

- Functions:

  - Start, authenticate, and terminate sessions.

  - Control file and printer access.

  - Allow applications to exchange messages.

## 6. Key Takeaways

- Application Layer provides network services to applications.

- Presentation Layer handles data formatting, compression, and encryption.

- Session Layer manages dialogues between applications.

- Client-Server and Peer-to-Peer are two primary network models.

- HTTP/HTTPS manage web communication.

- SMTP, POP, IMAP handle email.

- DNS translates domain names into IP addresses.

- DHCP dynamically assigns IP addresses.

- FTP and SMB are used for file sharing.